05 Resonance

Build a series RLC circuit, keep the output voltage value of AC source unchanged, and adjust frequency of the source signal.

A Report By

Kibria Golam

2019380163

1. When current of resistor reaches maximum Imax, the resonance frequency is found.  When current of resistor reaches 0.707Imax the half power frequency is found.

Table 1

R=5-ohm, L=30mH, C= 1uF

| f/Hz |  |  | f1 | 896.463 | f0 | 909.726 | f2 | 922.988 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ir/A |  |  | i1 | 0.072 | i0 | 0.412 | i2 | 0.284 |  |

Table 2

R=50-ohm, L=30mH, C= 1uF

| f/Hz |  |  | f1 | 786.37 | f0 | 919 | f2 | 1051.63 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ir/A |  |  | i1 | 0.029 | i0 | 0.041 | i2 | 0.029 |  |

2. When voltage of resistor reaches maximum Umax , the resonance frequency is found.  When current of resistor reaches 0.707Imax the half power frequency is found.

Table 3

R=5-ohm, L=30mH, C= 1uF

| f/Hz |  |  | f1 | 892.826 | f0 | 906.089 | f2 | 919.351 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ur/A |  |  | U1 | 0.377 | U0 | 2.016 | U2 | 1.623 |  |

Table 4

R=50-ohm, L=30mH, C= 1uF

| f/Hz |  |  | f1 | 786.37 | f0 | 919 | f2 | 1051.63 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ur/A |  |  | U1 | 1.464 | U0 | 2.057 | U2 | 1.453 |  |

3. When voltage values on inductor and capacitor are the same, the resonance frequency is found. Record maximum voltage Umax on resistor, then found half power frequency.

Table 5

R=5-ohm, L=30mH, C= 1uF

| f/Hz |  |  | f1 | 890.989 | f0 | 904.252 | f2 | 917.514 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IR/A |  |  | IR 1 | 0.243 | IR0 | 0.389 | IR2 | 0.346 |  |
| UR/V |  |  | UR1 | 1.216 | UR0 | 1.943 | UR2 | 1.731 |  |
| UL/V |  |  | UL1 | 41.243 | UL0 | 67.218 | UL2 | 60.752 |  |
| UC/V |  |  | UC1 | 42.858 | UC0 | 67.892 | UC2 | 59.647 |  |

Table 6

R=50-ohm, L=30mH, C= 1uF

| f/Hz |  |  | f1 | 771.28 | f0 | 903.91 | f2 | 1036.53 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IR/A |  |  | IR 1 | 0.027 | IR0 | 0.041 | IR2 | 0.031 |  |
| UR/V |  |  | UR1 | 1.37 | UR0 | 2.062 | UR2 | 1.527 |  |
| UL/V |  |  | UL1 | 4.203 | UL0 | 7.222 | UL2 | 6.099 |  |
| UC/V |  |  | UC1 | 5.574 | UC0 | 7.223 | UC2 | 4.65 |  |